Fantastic report on

MT_1

Distribution:

| Name | Function | 0.1 | 0.5 | 1.0 |
| --- | --- | --- | --- | --- |
| N.N.-999 | Masterking Group Manager (MGM) |  |  |  |
| N.N.-998 | Product Ontwikkelt Manager (POM) |  |  |  |
| N.N.-1 | (AS) |  |  |  |
| N.N.-2 | (DS) |  |  |  |
| N.N.-3 | (BS) |  |  |  |
| T.P. | Supervisor All Coding |  |  |  |

# Table of Contents:

Table of Contents:	2

Anchor Scan Parameters: (Bookmark1)	3

Graphics: (Bookmark2)	4

Peak List: (Bookmark 3)	5

Identified Patterns List: (Bookmark4)	7

Plot of Identified Phases: (Bookmark 5)	7

Document History: (Bookmark 6)	8

# Anchor Scan Parameters: (Bookmark1)

Dataset Name	MT.4

File name	C:\XRD Data\DATA FY16-17\Clients\Caldwell\MT.4.xrdml

Comment	Configuration=Configuration1, Owner=User-1, Creation date=2/15/2017 1:40:50 PM

Goniometer=PW3050/60 (Theta/Theta); Minimum step size 2Theta:0.001; Minimum step size Omega:0.001

Sample stage=Reflection-Transmission Spinner PW3064/60; Minimum step size Phi:0.1

Diffractometer system=XPERT-PRO

Measurement program=C:\PANalytical\Data Collector\Programs\Mineral Scan 2.xrdmp, Identifier={5A8E73D7-76AF-4A2E-AF38-7583B6D586B4}

Batch program=C:\PANalytical\Data Collector\Programs\1 Mineral Scan - 40 min.xrdmp, Identifier={8D4652C3-0312-4229-A694-F5EC0678D15E}

42 minute 5 - 70 degrees

PHD Lower Level = 6.68 (keV), PHD Upper Level = 12.88 (keV)

Measurement Start Date/Time	3/10/2017 1:16:27 PM

Operator	xrdlab

Raw Data Origin	XRD measurement (*.XRDML)

Scan Axis	Gonio

Start Position [°2θ]	5.0042

End Position [°2θ]	69.9882

Step Size [°2θ]	0.0080

Scan Step Time [s]	40.0050

Scan Type	Continuous

PSD Mode	Scanning

PSD Length [°2θ]	2.12

Offset [°2θ]	0.0000

Divergence Slit Type	Fixed

Divergence Slit Size [°]	0.2500

Specimen Length [mm]	10.00

Measurement Temperature [°C]	25.00

Anode Material	Cu

K-Alpha1 [Å]	1.54060

Generator Settings	40 mA, 45 kV

Diffractometer Type	0000000011071336

Diffractometer Number	0

Goniometer Radius [mm]	240.00

Dist. Focus-Diverg. Slit [mm]	100.00

Incident Beam Monochromator	No

Spinning	Yes

# Graphics: (Bookmark2)

# Peak List: (Bookmark 3)

| Pos. [°2θ] | Height [cts] | FWHM Left [°2θ] | d-spacing [Å] | Rel. Int. [%] | Tip Width | Matched by |
| --- | --- | --- | --- | --- | --- | --- |
| 6.2194 | 89.12 | 0.5038 | 14.19972 | 0.39 | 0.6046 |  |
| 12.4187 | 3776.39 | 0.0630 | 7.12173 | 16.66 | 0.0756 |  |
| 12.5641 | 1230.83 | 0.0472 | 7.03967 | 5.43 | 0.0567 |  |
| 13.6837 | 288.86 | 0.0630 | 6.46606 | 1.27 | 0.0756 |  |
| 13.9089 | 438.45 | 0.0630 | 6.36189 | 1.93 | 0.0756 |  |
| 19.1183 | 2078.27 | 0.0945 | 4.63853 | 9.17 | 0.1134 | 00-008-0304 |
| 19.8893 | 82.63 | 0.0945 | 4.46042 | 0.36 | 0.1134 |  |
| 20.4238 | 80.02 | 0.1260 | 4.34488 | 0.35 | 0.1511 |  |
| 20.9009 | 4302.36 | 0.0472 | 4.24676 | 18.99 | 0.0567 |  |
| 22.0577 | 396.66 | 0.1102 | 4.02659 | 1.75 | 0.1322 |  |
| 22.3533 | 104.93 | 0.0945 | 3.97401 | 0.46 | 0.1134 |  |
| 23.0492 | 130.78 | 0.1260 | 3.85557 | 0.58 | 0.1511 |  |
| 23.2363 | 194.80 | 0.0472 | 3.82495 | 0.86 | 0.0567 |  |
| 23.6499 | 312.78 | 0.1260 | 3.75899 | 1.38 | 0.1511 |  |
| 24.0260 | 213.12 | 0.0945 | 3.70098 | 0.94 | 0.1134 |  |
| 24.3463 | 226.98 | 0.1102 | 3.65301 | 1.00 | 0.1322 |  |
| 24.9042 | 1921.75 | 0.0551 | 3.57243 | 8.48 | 0.0661 |  |
| 25.2115 | 739.25 | 0.0630 | 3.52957 | 3.26 | 0.0756 |  |
| 25.6927 | 2529.52 | 0.0551 | 3.46455 | 11.16 | 0.0661 | 00-008-0304 |
| 26.6936 | 22660.75 | 0.0480 | 3.33687 | 100.00 | 0.0576 |  |
| 27.1126 | 359.91 | 0.0768 | 3.28625 | 1.59 | 0.0922 |  |
| 27.5076 | 4119.22 | 0.0672 | 3.23995 | 18.18 | 0.0806 |  |
| 27.9754 | 3212.56 | 0.0672 | 3.18683 | 14.18 | 0.0806 |  |
| 29.5027 | 112.67 | 0.1152 | 3.02523 | 0.50 | 0.1382 |  |
| 30.5035 | 251.45 | 0.0960 | 2.92822 | 1.11 | 0.1152 |  |
| 30.7874 | 153.08 | 0.1536 | 2.90186 | 0.68 | 0.1843 |  |
| 31.0045 | 235.79 | 0.0576 | 2.88204 | 1.04 | 0.0691 |  |
| 31.4403 | 38.63 | 0.2688 | 2.84307 | 0.17 | 0.3226 |  |
| 32.2516 | 708.24 | 0.1056 | 2.77338 | 3.13 | 0.1267 | 00-008-0304 |
| 33.0312 | 79.14 | 0.1536 | 2.70968 | 0.35 | 0.1843 |  |
| 33.9811 | 1321.37 | 0.0576 | 2.63608 | 5.83 | 0.0691 | 00-008-0304 |
| 35.0588 | 108.00 | 0.2304 | 2.55748 | 0.48 | 0.2765 |  |
| 35.9304 | 275.92 | 0.3456 | 2.49741 | 1.22 | 0.4147 |  |
| 36.5944 | 1447.26 | 0.0576 | 2.45360 | 6.39 | 0.0691 | 00-008-0304 |
| 37.0334 | 97.49 | 0.1152 | 2.42552 | 0.43 | 0.1382 |  |
| 37.7332 | 170.98 | 0.1920 | 2.38213 | 0.75 | 0.2304 |  |
| 38.5463 | 142.67 | 0.3072 | 2.33373 | 0.63 | 0.3686 |  |
| 39.5194 | 1212.33 | 0.0480 | 2.27848 | 5.35 | 0.0576 |  |
| 40.3421 | 613.94 | 0.0672 | 2.23389 | 2.71 | 0.0806 |  |
| 41.6387 | 409.32 | 0.1536 | 2.16728 | 1.81 | 0.1843 |  |
| 42.5072 | 1109.85 | 0.0768 | 2.12498 | 4.90 | 0.0922 |  |
| 45.3619 | 324.94 | 0.0960 | 1.99767 | 1.43 | 0.1152 |  |
| 45.8363 | 607.54 | 0.0672 | 1.97809 | 2.68 | 0.0806 |  |
| 47.1614 | 34.67 | 0.4608 | 1.92555 | 0.15 | 0.5530 |  |
| 48.9146 | 57.18 | 0.3072 | 1.86056 | 0.25 | 0.3686 |  |
| 49.5712 | 265.35 | 0.1152 | 1.83744 | 1.17 | 0.1382 | 00-008-0304 |
| 50.1925 | 1795.99 | 0.0768 | 1.81614 | 7.93 | 0.0922 |  |
| 50.5762 | 307.37 | 0.0576 | 1.80326 | 1.36 | 0.0691 | 00-008-0304 |
| 50.8025 | 826.26 | 0.0672 | 1.79576 | 3.65 | 0.0806 |  |
| 52.7614 | 279.11 | 0.1920 | 1.73361 | 1.23 | 0.2304 | 00-008-0304 |
| 54.9084 | 603.50 | 0.0672 | 1.67079 | 2.66 | 0.0806 |  |
| 55.3685 | 255.92 | 0.0768 | 1.65799 | 1.13 | 0.0922 |  |
| 56.1397 | 34.87 | 0.3072 | 1.63703 | 0.15 | 0.3686 |  |
| 56.6434 | 102.83 | 0.1920 | 1.62366 | 0.45 | 0.2304 |  |
| 58.7464 | 42.47 | 0.3072 | 1.57045 | 0.19 | 0.3686 |  |
| 59.6581 | 275.31 | 0.1728 | 1.54861 | 1.21 | 0.2074 |  |
| 60.0041 | 1373.78 | 0.0768 | 1.54050 | 6.06 | 0.0922 |  |
| 61.2711 | 69.32 | 0.3072 | 1.51165 | 0.31 | 0.3686 |  |
| 62.3815 | 25.75 | 0.2304 | 1.48738 | 0.11 | 0.2765 |  |
| 64.0690 | 319.69 | 0.0672 | 1.45222 | 1.41 | 0.0806 | 00-008-0304 |
| 64.2589 | 321.41 | 0.0768 | 1.44838 | 1.42 | 0.0922 | 00-008-0304 |
| 65.3640 | 169.46 | 0.0768 | 1.42654 | 0.75 | 0.0922 |  |
| 66.1059 | 39.88 | 0.4608 | 1.41232 | 0.18 | 0.5530 |  |
| 67.7853 | 725.02 | 0.1056 | 1.38136 | 3.20 | 0.1267 |  |
| 68.1825 | 1002.21 | 0.0576 | 1.37428 | 4.42 | 0.0691 |  |
| 68.3458 | 1037.67 | 0.0768 | 1.37139 | 4.58 | 0.0922 |  |

# Identified Patterns List: (Bookmark4)

| Visible | Ref.Code | Score | Compound Name | Displ.[°2θ] | Scale Fac. | Chem. Formula |
| --- | --- | --- | --- | --- | --- | --- |
| * | 00-008-0304 | 24 | Uranium Silicate Hydroxide | 0.000 | 0.040 | U ( Si O4 ) ( O H ) |

# Plot of Identified Phases: (Bookmark 5)

# Document History: (Bookmark 6)

Insert Measurement:

- File name = "MT.4.xrdml"

- Modification time = "3/31/2017 11:09:35 AM"

- Modification editor = "xrdlab"

Default properties:

- Measurement step axis = "None"

- Internal wavelengths used from anode material: Copper (Cu)

- Original K-Alpha1 wavelength = "1.54060"

- Used K-Alpha1 wavelength = "1.54060"

- Original K-Alpha2 wavelength = "1.54443"

- Used K-Alpha2 wavelength = "1.54443"

- Original K-Beta wavelength = "1.39225"

- Used K-Beta wavelength = "1.39225"

- Irradiated length = "10.00000"

- Receiving slit size = "0.10000"

- Step axis value = "0.00000"

- Offset = "0.00000"

- Sample length = "10.00000"

- Modification time = "3/31/2017 11:09:35 AM"

- Modification editor = "xrdlab"

Interpolate Step Size:

- Initial Scan Range = 5.00418 - 69.99430

- Initial Step Size = 0.00836

- Derived Step Size = 0.00800

- Use Derived Step Size = "Yes"

- Modification time = "3/31/2017 11:09:35 AM"

- Modification editor = "PANalytical"

Determine Background:

- Add to net scan = "Nothing"

- User defined intensity = "-5"

- Correction method = "Automatic"

- Bending factor = "5"

- Minimum significance = "0.7"

- Minimum tip width = "0"

- Maximum tip width = "1"

- Peak base width = "2"

- Use smoothed input data = "Yes"

- Granularity = "20"

- Modification time = "2/22/2001 10:17:43 AM"

- Modification editor = "PANalytical"

Search Peaks:

- Minimum significance = "2"

- Minimum tip width = "0.01"

- Maximum tip width = "1"

- Peak base width = "2"

- Method = "Minimum 2nd derivative"

- Modification time = "2/20/2001 11:55:18 AM"

- Modification editor = "PANalytical"

Search & Match:

- Allow pattern shift = "No"

- Auto residue = "Yes"

- Data source = "Profile and peak list"

- Demote unmatched strong = "Yes"

- Multi phase = "Yes"

- Restriction set = "Minerals"

- Restriction = "Restriction set"

- Subset name = ""

- Match intensity = "Yes"

- Two theta shift = "0"

- Identify = "Yes"

- Max. no. of accepted patterns = "5"

- Minimum score = "27"

- Min. new lines / total lines = "40"

- Search depth = "6"

- Minimum new lines = "3"

- Minimum scale factor = "0.06"

- Intensity threshold = "0"

- Use line clustering = "Yes"

- Line cluster range = "1.5"

- Search sensitivity = "1.8"

- Use adaptive smoothing = "Yes"

- Smoothing range = "1.5"

- Threshold factor = "3"

- Match Threshold = "0"

- Modification time = "2/5/2001 11:17:06 AM"

- Modification editor = "PANalytical"

Strip K-α2:

- Method = "Rachinger"

- Intensity ratio = "0.5"

- Delta wavelength ratio = "0"

- Modification time = "3/31/2017 11:10:00 AM"

- Modification editor = "PANalytical"